| ## Dialdecoder ### DTMF SIGNAL READER. MODEL:TA-BD210.. OPERATION MANUAL TRONIC ACE TECHNOLOGY INC. http://www.unitone.com.tw. | ※※ INTRODUCTIONS ※※ The Dialdecoder is the product name of DTMF SIGNAL READER ( or DTMF Decoder ). The DTMF is the abbreviation of Dual Tone Multi- Frequency and is also known as the Touch Tone. They are the combination audible sounds you hear when you press keys on the phone, cellular phone or walkie talkie. The DTMF Decoder ( Tone Decoder ) can read the DTMF signals and decode them at the LCD display. The user can read out signals as 1, 2, 3, 4, 5, 6, 7, 8, 9, 0, or A, B, C, D. The device is often used to decode the dialing tone number which is recorded at the tape of telephone recorder. ＜2＞ |  |  | ※※ APPLICATIONS ※※ - Helps prosecutors, investigators or policemen collecting evidence for law enforcement purpose. - Helps private detectives, personal users register or decode the outgoing call numbers. - Laboratories use it as the tester to development new products. - Retailers use it to test and demonstrate products. - The user of telephone recorder. ＜3＞. | ※※ FUNCTIONS ※※ - Can read out the DTMF signals accurately and quickly and display the digit or signal at LCD screen. - If the product is tapped at the telephone line, it can read out the dialing number. - 3 decoded numbers can be stored. - "BEEP" reminding sound at pressing function keys. - 2 optional ways for audio input. - Steel extendible stand. ＜4＞ |
| --- | --- | --- | --- | --- | --- |
| ※※ LOCATION OF CONTROLS ※※ ＜5＞ | - “DC IN” : DC Power Input Jack. - POWER : Power Switch ( left for ON and right for OFF ). - “MIC” : Microphone Jack. - AUDIO IN : Touch Tone Input Jack. - Buzzer : BEEP sound output buzzer. - CLEAR : The key for cleaning digits ( number ) shown at LCD screen. - STORE : The key for storing the decoded numbers shown at LCD screen. - RECALL : The key to re-call the stored numbers. - LCD screen : 2 lines for showing the status and digits ( numbers ) decoded. ＜6＞ |  |  | - Steel Stand : can be extended to support the unit, so that the user can read the number clearly.. - Battery Cover : for inserting one 9V battery. - AC/DC Adaptor : Can transform and change the power from AC to DC power. - Audio Cord : Plug it into the the jack of unit’s “AUDIO IN” jack for inputting the DTMF signal to the unit. ＜7＞ |  | - 9V battery. ※※ ACCESSORIES ※※ The package includes following elements : - Unit : TA-BD210. - Audio Cord. - 9V battery. - Operation Manual. ※※ OPTIONAL ACCESSORIES ※※ - Power Adaptor : 9VDC input power; 200mA - Deluxe Steel Box. ＜8＞ |
| ※※ INSTALLATIONS ※※ - Supply the power : Use one 9V battery as the power or insert the AD/DC adaptor to supply the power from AC power outlet --- - Use internal 9V battery as the power : Remove the battery cover by sliding it in the opposite direction of arrow sign. Clip one 9V battery, place it into manger. Then fix it by battery cover. - Use external power : Note that the AC power level is within the limitation of adaptor’s input power level. Then plug in the Adaptor to the AC outlet and the unit’s ＜9＞ |  | jack respectively. - Place the unit at the desk by extend the Steel Stand. ※※ OPERATIONS ※※ - POWER THE UNIT Push the power to turn on the power. You will hear ”BEEP” sound and the LCD screen shows ”UNITONE DIALDECODER”, ”DETECTNG”. Then you will hear “BEEP, BEEP” sounds and the screen shows ”DECODE1. It means that the Dialdecoder is under detection. ＜10＞ | - PICK UP THE DTMF SIGNAL - From the build-in microphone : Put the audio sound speaker ( at recorder, telephone set or touch-tone dialer ) close to the unit’s microphone, play out the sound. Note that the environmental sound level cannot be too loud. Otherwise, it will interfere with the detection. - From the AUDIO IN JACK : Connect the audio cord between the unit and the audio source, then play out the audio sound. - CLEAR THE SCREEN Press “ CLEAR” key can clear the digit shown at LCD display. ＜11＞ |  | - CLEAR ALL DETECTED DIGIT Press “CLEAR” key can clear the digit shown at screen. - STORE DETECTED NUMBER Press ”STORE” key can store the number displayed at screen. - RECALL STORED NUMBER Press ”RECALL” key can call out the stored number. ＜12＞ |  |
| ※※ SPECIFICATIONS ※※ DIMENSION : 14cm × 7cm × 2.5cm UNIT WEIGHT : 100g PACKAGE WEIGHT : 300g POWER : 9V CONSUMPTION POWER CURRENT : 75mA DTMF SIGNAL 偵測容許誤差度： 偵測速限： ＜13＞ |  | ※※ NOTES ※※ - The product will generate “BEEP” sound to confirm each operation for the user when he press any function key from the product. - One of following factors may cause unsuccessful decoding : - The sound is not DTMF ( tone ) signal : Sometime it sounds like tone signal but actually it is not. - The tone signal was distorted : The old or low quality telephone set, tone dialer may generate incorrect tone signal that exceeds the tolerance of DTMF. ＜14＞ | 3、 Tone signal level was too low 4、 The tone signal was not clear : the tape is too old. Try to solve above problems or ask your vendor to get correct solution. - Keep the product in box when you do not want to use it. Avoid humidity and violent vibration. - Do not switch off the power and power it on immediately within 5 seconds. Otherwise, the product may shut down since the micro processor cannot react the operation in time. ＜15＞ |  | In such a situation, turn off the power and turn it on after 5 seconds.. - If the recording and play is not at the same recorder, the recorded tone signal may be distorted. So that it cannot be decoded. ＜16＞ |  |